Abstract:

Collected at Laboratory of Neuro-engineering and Rehabilitation, Tianjin University, this database consists of EEG recordings from 28 subjects with normal or corrected to normal vision, used to study the asymmetric visual evoked potentials. The data is divided into six sessions in which the displayed stimuli and experimental blocks differ.

Data Description:

Recordings, grouped into 28 cases, which collected from 28 subjects (age from 20-28, 16 females).

Each case (Sub01, Sub02, etc.) contains six .edf files from a single subject. In order to protect the privacy of the subjects, all protected health information (PHI) in the original .edf files has been replaced with surrogate information in the files provided here. The .edf files contain nearly two minutes of task state data and about 1 minute of resting state data before and after the experiment.

EEG was recorded using a Neuroscan Synamps2 system with 21 electrodes located in the positions following the 10/20 system. The reference electrode was put in the central area at the top of the head and the ground electrode was put on the frontal lobe. The recording signals were bandpass-filtered at 0.1-100Hz, notch-filtered at 50Hz, digitized at a rate of 1000 Hz. The data has been pre-processed before uploading. The signals were extracted from EEG data of 21 channels (P7, P5, P3, P1, PZ, P2, P4, P6, P8, PO7, PO5, PO3, POZ, PO4, PO6, PO8, CB1, O1, OZ, O2, CB2) and were down-sampled at 250 Hz. The data in the .edf file has the 22th channel, which indicates the event triggers. There are three types of event triggers: ‘1’ , ‘2’ and the values greater than 50. Other samples in this channel indicate that they are not event onset.

The file RECORDS contains a list of all 168 .edf files included in this collection, and the file Session_INFO.txt list the block id of 6 session and the character displayed in a single .edf file. Sessions with odd index have the same character list, so do even index. A complete block consist of two consecutive sessions.

Experimental Procedures

Thirty-two alphanumeric characters (‘A’ to ‘Z’ and ‘1’ to ‘6’) were encoded by a new space-code division multiple access (SCDMA). in which the left and right lateral visual stimuli constituted two parallel spatial channels while two different lateral visual stimuli sequences made up the basic communication codes ‘0’ and ‘1’. Specifically, the ‘left-right’ stimulus sequence, which lasted 200 ms, was regarded as code ‘0’, while ‘right-left’ stimulus was coded ‘1’. Thirty-two different code sequences were created using 5 bits in this study, which were arbitrarily allocated to different characters. Specifically, character ‘A’ was encoded by ‘01100’. In spelling, the lateral visual stimuli would be presented simultaneously for all characters with different sequences. To obtain a reliable output, the same code sequence was repeated 6 times for the offline spelling. The annotation in .edf file provide the order of target character. The character ‘A’ ~ ‘D’ were marked with 52~55, ‘E’ ~ ‘H’ were marked with 62~65, and the like. The basic communication codes ‘0’ and ‘1’ were marked with 1 and 2. All of marks can be find in last channel of data, in which channel label is ‘events’.

Background

Traditional visual brain–computer interfaces (BCIs) preferred to use large-size stimuli to attract the user’s attention and elicit distinct electroencephalography (EEG) features. However, using stronger visual stimuli could cause visual fatigue and other adverse symptoms to users. Therefore, it’s imperative for visual BCIs to use small and inconspicuous visual stimuli to code characters. We developed a new BCI speller based on miniature asymmetric visual evoked potentials (aVEPs), and demonstrated the feasibility of using very small and inconspicuous visual stimuli to implement an efficient BCI system. Now we share the dataset so that the noise removal and decoding of aVEP signals can be further studied.

When using this resource, please cite the

Collected at Laboratory of Neuroengineering and Rehabilitation, Tianjin University, this database consists of EEG recordings from 28 subjects with normal or corrected to normal vision, used to study the asymmetric visual evoked potentials. The data is divided into six sessions in which the displayed stimuli and experimental blocks differ.

Xu M;Xiao X;Wang Y;Qi H;Jung TP;Ming D; “A Brain-Computer Interface Based on Miniature-Event-Related Potentials Induced by Very Small Lateral Visual Stimuli.” IEEE Transactions on Bio-Medical Engineering, U.S. National Library of Medicine, https://pubmed.ncbi.nlm.nih.gov/29683431/.